# The following guide was downloaded from http://support.touch-base.com/Documentation/50245/Quick-installation-guide on 05/07/17

# Quick installation guide

Welcome to the Mac OS X Quick Installation Guide which covers the key elements for installing the UPDD Mac OS X version 6 driver. At the time of writing this driver supports Mac OS X 10:7 thro’ 10:13.

A full UPDD driver installation document is available here and contains comprehensive installation and usage instructions and should be referenced if installation fails or the touch screen does not function as expected and for a greater understanding of the UPDD driver software.

The software is delivered via email as a HTTP download link to a Mac OS X disk image container file updd_06_nn_nn.dmg. The actual name of the .dmg file reflects the version of the driver being installed, e.g.updd_06_00_1.dmg.

The .dmg file will expand to a package file updd.pkg (this step may be invoked automatically when downloading the .dmg file).



To install the software double click on the package file and follow the instructions on the install dialog until the ‘Installation has completed’ screen is seen. During the installation procedure you will be prompted to enter your password.



In most cases the installer will only install the touch driver, related driver utilities and gesture extension but in other cases it may also install additional driver add-ons such as the TUIO server. The latest Gesture and TUIO software can always be downloaded from the associated documentation.

Following install the touch screen should be working and the point of touch activated under the stylus. If the touch is working but the point of touch is not calibrated then invoke the configure procedure. If there is no response try rebooting otherwise refer to the troubleshooting document.

The touch hardware and UPDD software can initially be tested with the test utility.

#### Menu Bar Items

There will be Menu Bar item for each UPDD component installed (in some OEM installs these may be disabled):

Selecting the menu bar icon will list the menu options:

| Daemon | Gestures | TUIO |
| --- | --- | --- |

With UPDD version 6, applications that connect to the driver via the driver's API, such as the three above, will receive notification if the connection to the API drops, implying the driver is not available. In this instance the menu bar icons will show an exclamation mark and touch is unlikely to be active, such as the driver's menu bar icon:

- driver disconnected / connected notification

The UPDD utilities programs are placed in the Mac’s Utilities or the UPDD's Application folder. There are a number of utilities related to the driver and one each for gestures and TUIO server when they are installed (either by the driver installer or separately).

| Driver | Gestures | TUIO |
| --- | --- | --- |
| Calibration; Test; Uninstall. | Settings. | Settings. |

Notes: Uninstall and the UPDD Command Line Utility, are actually held in the UPDD application folder /library/Application Support/updd.

Following installation the touch should be working and calibrated, if not please review the troubleshooting guide.

Test utility

A simple test utility exist to test the touch mouse interface and the UPDD API and TUIO Server data input.

#### Calibration

#### If the registered touch point is offset from the point of touch, or the touch is not associated with the correct monitor, then the touch screen will need to be calibrated with the video area. The simplest way to achieve this is to invoke the Configure option from the Daemon menu.

#### System Interfaces

A number of System Interfaces are used to post touch data into the Mac OS X system and applications as described below:

| Interface |  | Description |
| --- | --- | --- |
| Driver |  | The driver reads all touch data from the touch screen but only posts single touches directly into the system such that the touch screen acts like a mouse type device. All touch data received from the device is made available on the driver’s Application Programmers Interface (API). |
| Docs |  | The complete documentation is here. |
| Settings |  | The driver settings can be adjusted in the Command Line Utility. |
| Updates |  | The UPDD Daemon, About box shows the version in use. Contact the driver supplier for any updates. |
| Gestures |  | The gesture application receives touch data from the driver API, performs the action associated with the incoming gesture and then posts the touch data into the OS as native touch events. |
| Docs |  | The complete documentation is here. |
| Settings |  | The gesture settings can be adjusted in the gesture settings dialog. Note – The ‘Click nearest clickable UI element’ in the Taps and Presses dialog is a useful setting to enable if you are using the touchscreen to select small buttons, such as the minimise, maximise and close buttons in standard application dialogs as these were not designed for touch screen usage. |
| Updates |  | The gestures menu bar item ‘Change History’ will show the version in use. Release notes can be found here. The latest software is available via a link in the Gesture documentation. |
| TUIO |  | The TUIO Server receives touch data from the driver API and posts the touch data to TUIO Client applications. This software is only required if you are running TUIO client applications. |
| Docs |  | The complete documentation is here. |
| Settings |  | The TUIO settings can be adjusted in the TUIO settings dialog. |
| Updates |  | The TUIO Menu Bar menu item ‘Change History’ will show the version in use. Release notes can be found here. The latest software is available via a link in the TUIO documentation. |